## Raw lecture notes (the source material — DO NOT copy verbatim)

```

To teach robotics from scratch, we must move away from "pencil-and-paper" math and toward **algorithmic thinking**. This course is designed to take an engineer from zero to building the mathematical backend of a navigation system.

---

## Unit 1: Vectors and the Geometry of Space

In robotics, a vector isn't just a list of numbers; it is a **directed arrow** representing a force, a velocity, or a position in the world.

### 1.1 Linear Combinations and Span

If a robot has two motors (thrusters), each motor provides a force vector $v_1$ and $v_2$. The set of all possible directions the robot can move is the **Span** of those vectors.

* **Equation:** $v_{total} = c_1 v_1 + c_2 v_2$

* **Engineering Context:** If $v_1$ and $v_2$ are "Linearly Dependent" (pointing in the same direction), your robot is stuck moving in only one dimension.

* **Example:** A drone with four rotors. Each rotor produces a thrust vector. To hover, the linear combination of these four vectors must exactly oppose the gravity vector $g$.

---

## Unit 2: Solving the Robot’s State ($Ax = b$)

The most fundamental problem in robotics is: "I know where I want to be ($b$), and I know how my motors work ($A$). What signals ($x$) do I send?"

### 2.1 The Matrix as a System

We represent $n$ equations with $n$ unknowns as $Ax = b$.

* **The Problem:** Solving $x = A^{-1}b$ is computationally expensive and numerically unstable for large matrices.

* **The Engineering Solution: LU Decomposition.**

We decompose $A$ into a **Lower** ($L$) and **Upper** ($U$) triangular matrix.

$$A = LU$$

1. Solve $Ly = b$ (Forward substitution).

2. Solve $Ux = y$ (Backward substitution).

### 2.2 Row Echelon Form and Pivoting

When a computer solves these, it uses **Gaussian Elimination**. To prevent "division by zero" or massive rounding errors, we use **Partial Pivoting** (swapping rows to put the largest number on the diagonal).

---

## Unit 3: Spatial Transformations (SE(2) and SE(3))

A robot must constantly translate data from its "Eye" (Camera) to its "Hand" (Gripper).

### 3.1 Rotation Matrices ($SO(n)$)

To rotate a vector in 2D by angle $\theta$:

$$R(\theta) = \begin{bmatrix} \cos\theta & -\sin\theta \\ \sin\theta & \cos\theta \end{bmatrix}$$

* **Property:** $R^T = R^{-1}$ (Orthonormal). This is vital because it means to "un-rotate" something, you just transpose the matrix—which is computationally free!

### 3.2 Homogeneous Transformation Matrices

To move and rotate simultaneously, we use a $4 \times 4$ matrix for 3D space. This allows us to treat "Translate then Rotate" as a single matrix multiplication.

$$T = \begin{bmatrix} R & t \\ 0 & 1 \end{bmatrix}$$

Where $R$ is a $3 \times 3$ rotation matrix and $t$ is a $3 \times 1$ translation vector.

* **Application:** If a camera sees a ball at coordinates $P_{camera}$, and you know the transformation from the robot base to the camera $T_{base}^{cam}$, the ball's position in the real world is:

$$P_{world} = T_{base}^{cam} \cdot P_{camera}$$

---

## Unit 4: Handling Noise (Least Squares)

In the real world, $Ax = b$ usually has **no solution** because sensor noise makes the equations inconsistent. We instead find the $x$ that minimizes the error.

### 4.1 The Normal Equation

To find the "Best Fit" for an overdetermined system (more sensors than variables):

1. Define the error (residual): $r = b - Ax$.

2. Minimize the square of the error: $||Ax - b||^2$.

3. **The Solution:** $$x = (A^T A)^{-1} A^T b$$

### 4.2 Recursive Least Squares (RLS)

Robots cannot wait for all data to arrive; they must update their "guess" as new data streams in.

* **Example:** A self-driving car estimating the lane line position. As each new frame of video comes in, it updates the previous $x$ estimate using a **Weighting Matrix** (The foundation of the Kalman Filter).

---

## Unit 5: Stability and SVD

How do we know if a robot is about to tip over or if a robotic arm is reaching a "singularity" (a point where it breaks)?

### 5.1 Eigenvalues and Eigenvectors

If $Ax = \lambda x$, then $x$ is an eigenvector and $\lambda$ is an eigenvalue.

* **Engineering Use:** In drone flight control, if the eigenvalues of the system matrix have a real part greater than zero, the drone’s oscillations will grow until it crashes. **Stability = Negative Eigenvalues.**

### 5.2 Singular Value Decomposition (SVD)

SVD decomposes any matrix (even non-square ones) into $A = U\Sigma V^T$.

* **The "Condition Number":** $\sigma_{max} / \sigma_{min}$.

* **The Singularity Problem:** If the smallest singular value $\sigma_{min}$ is near zero, the robot is in a "singular configuration" (e.g., an arm fully locked straight). The robot loses a degree of freedom, and the math "explodes."

---

## Unit 6: Computational Implementation (The Lab)

For your application, the user should implement these algorithms without using high-level libraries like `numpy.linalg.solve` initially.

### Project: The LIDAR Scan Matcher

1.  **Input:** Two sets of points $(X, Y)$ from a LIDAR sensor taken 1 second apart.

2.  **Task:** Find the Rotation $R$ and Translation $t$ that aligns the two sets.

3.  **Algorithm:**

* Find the centroid of both point clouds.

* Subtract the centroid (Centering).

* Calculate the **Cross-Covariance Matrix** $W$.

* Use **SVD** on $W$ to extract the optimal Rotation Matrix.

* Result: The user has just programmed **Odometry**—the ability for a robot to know it moved just by "looking" at the world.

---

## Essential Hardware/Software Stack for this Course

* **Language:** Julia (Recommended for its mathematical syntax) or C++ (Industry standard).

* **Visualization:** Use a library like `MeshCat.jl` or `Matplotlib` to animate the vectors.

* **Key Textbook Reference:** *Linear Algebra and Learning from Data* by Gilbert Strang.